Audio Adjustments for ShoreTel 210, 210A, 230, 530, and 560 phones

The audio levels for the handset, headset and speaker are set to default values when the phone boots.  These values can be overridden by configuration file settings.  The phone supports multiple configuration files, and the last setting encountered in a configuration file wins.  The base configuration file for each phone type is created by the installer, so although new values may be set in these files, they will be lost when the system is upgraded.  Customized audio levels should be set in customized configuration files rather than the base configuration files.  Below is a table showing phone name, model and file names:

| ShorePhone Name | Name on Top of Phone | Model Name on Barcode Label Underside of Phone | Base Configuration File Name | Custom File Name c:\Inetpub\ftproot\ |
| --- | --- | --- | --- | --- |
| 110 | 110 | S0 | shore_s0.txt | S0custom.txt |
| 210 | 210 | S1 | shore_s1.txt | s1custom.txt |
| 210A | 210 | S1A | shore_s1a.txt | s1custom.txt |
| 530 | 530 | S2 | shore_s2.txt | s2custom.txt |
| 560 | 560 | S6 | shore_s6.txt | s6custom.txt |
| 212k | 212k | S12 | Shore_s12.txt | S12custom.txt |
| 230 | 230 | 230 | shore_sev.txt | sevcustom.txt |
| 560G | 560G | S6G | shore_s6g.txt | s6gcustom.txt |

There are four sets of audio levels that can be custom configured:  headset, handset, speaker and ringer.  Below are the default values for each type:

Prior to ST6.1:

HandsetLevels    5157,183,258,365,516,728,1029,1631,2303,3254,4596,917,13

HeadsetLevels    6144,183,258,365,516,728,1029,1631,2303,3254,4596,1375,13

RingerLevels     130,258,410,649,1029,1631,2584,4096,6492,10284

SpeakerLevels    4096,258,410,649,1029,1631,2584,4096,6492,10289,16306,13

Here are the parameter definitions:

HandsetLevels	TxGain, RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7, RxGain8, RxGain9, RxGain10, Side Tone, Handset DTMF Attenuation

HeadsetLevels	TxGain, RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7, RxGain8, xGain9, RxGain10, Side Tone, Headset DTMF Attenuation

RingerLevels	RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7 RxGain8, RxGain9, RxGain10

SpeakerLevels	TxGain, RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7, RxGain8, RxGain9, RxGain10, Speaker DTMF Attenuation

ST6.1:

HandsetLevels    5157,183,258,365,516,728,1029,1631,2303,3254,4596,917,13,13

HeadsetLevels    6144,183,258,365,516,728,1029,1631,2303,3254,4596,1375,13,13

RingerLevels     130,258,410,649,1029,1631,2584,4096,6492,10284

SpeakerLevels    4096,258,410,649,1029,1631,2584,4096,6492,10289,16306,13,13

Here are the parameter definitions:

HandsetLevels	TxGain, RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7, RxGain8, RxGain9, RxGain10, Side Tone, Handset DTMF Attenuation, Call Progress Attenuation

HeadsetLevels	TxGain, RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7, RxGain8, RxGain9, RxGain10, Side Tone, Headset DTMF Attenuation, Call Progress Attenuation

RingerLevels	RxGain1, RxGain2, RxGain3, RxGain4, RxGain5, RxGain6, RxGain7, RxGain8, RxGain9, RxGain10

SpeakerLevels	TxGain, RxGain1, RxGain2, RxGain3, RxGain4, RxGain5,RxGain6, RxGain7, RxGain8, RxGain9, RxGain10, Speaker DTMF Attenuation, Call Progress Attenuation

The "TxGain" parameter sets the level of the audio transmitted from the phone onto the network.  The "RxGain" values correspond to each of the 10 volume setting levels shown when one adjusts the volume on the phone.  "SideTone" is the audio picked up from the microphone and transmitted to the speaker (speakerphone, handset or headset speaker) that provides feedback to the user that the phone is working speaking.  Customer experience has shown that sometimes the RxGains are not high enough and need to be customized for an individual system.  SideTone gain is very subjective, and is sometimes lowered and sometimes raised.  DTMF Attenuation sets the receive DTMF level that the phone user will hear.

The Plantronics CS50 wireless headset provides its own side tone, plus inserts 20 ms delay between the headset and the phone, which causes some people to say they hear echo when using the Plantronics headset, and in this case headset side tone may need to be reduced.   When you speak, Plantronics does reduce the gain on the speaker by -24dB, but when you stop speaking, the gain is turned up, and you can hear the last little bit echoed in your ear.

For example, inserting the following line in "s2custom.txt" or "s6custom.txt" and rebooting the phone increases the headset volume settings 11 dB at the highest setting, which may be required in noisy environments:

Note:	ShoreTel 6.1 added an additional parameter. The examples below reflect the settings prior to that release

HeadsetLevels   6144,183,258,409,649,1028,1630,2899,6491,10288,14333,1375,13

The same levels could be used for "HandsetLevels", and also for the 210 phone.  Inserting the following line reduces the headset side tone by 9.5 dB, which some CS50 users prefer:  There were releases and opinions that 460 was found to be a better value for sidetone.

Note:	ShoreTel 6.1 added an additional parameter. The examples below reflect the settings prior to that release

HeadsetLevels    6144,183,258,365,516,728,1029,1631,2303,3254,4596,460,13

Minimum gain values are 0 and maximum are 32536.  Setting values very high may saturate the speaker and create poor sound quality.

N.B.:  You can change gains on an individual phone by changing the configuration files and then rebooting that phone.  But eventually all phones on the system will obtain these values when they eventually reboot.  We do not currently have the ability to individually configure ShoreTel IP phones.

| TX Levels |  |
| --- | --- |
| Gain | Value |
| Flat 0db | 4096 |
| 1db | 6144 |
| 2db | 8192 |
| 3db | 10240 |
| Max 4db | 12288 |