Configuring DHCP for ShoreTel IP 4XX Phones ShoreTel 14+

The ShoreTel server provides the IP phones with the latest application software and the

configuration information that enables the IP phone to be automatically added to the

ShoreTel system. The ShoreTel server’s address must be provided to the phone as a

vendor-specific option.

To set up DHCP option 156 for ShoreTel telephones:

- Step 1 Open DHCP Manager on your Microsoft DHCP server.

- Step 2 Right-click the DHCP server, and select Set pre-defined options.

NOTE If your organization is separated into separate subnets, make sure to select the proper subnet. For example, if you have a global organization and would like to configure the DHCP server to deliver the Spanish tones and cadences only to theIP phones in your office in Spain, you should make sure to select that particular subnet of users. If you do not specify the subnet, then all phones that boot from this DHCP server will receive Spanish tones and cadences.

- Step 3 Click Add.

- Step 4 Set Name to IP Phone Boot Server.

- Step 5 Set Data Type to String.

- Step 6 Set Code to 156 and add a description, if desired.

- Step 7 Navigate to the scope options and add option 156.

- Step 8 Set the value of option 156 to: ftpservers=ip_address, country=n, language=n, layer2tagging=n, vlanid=n

Here is an example if you server ip was 192.168.1.50

Configservers=192.168.1.10, ftpservers=192.168.1.50, country=n, language=n, layer2tagging=n, vlanid=n

Scope Option 4

Use Scope Option 4 to point the ShoreTel phones to an SNTP time server, if you do not use an internal one you can point them to the US Navy time server 192.5.41.41