L3 -> L2  00 DL_DATA_REQ        001 8001                    0001 SETUP

300 20a 00 240       00 00 00 00    	08 02 00 01 05 04 03 80 90 a2 18 03 a9 83 97 6c

07 00 80 36 39 30 30 34 70 06 80 34 36 35 34 30

SAPI

TEI

Control Field

PD (Protocol Discriminator) 0x08=Call Control, 0x03=Maintenance

CRV (Call Refernece Value)

Call Control Message

IE (Information Element)

Length

Element Content

Call Control Message:

Alerting			01

Call Processing		02

Connect			07

Connect Acknowledge	0F

Progress			03

Setup				05

User Info			20

Disconnect			45

Release			4D

Release Complete		5A

Restart				46

Restart Acknowledge		4C

Congestion Center		79

Facility			62

Status				7D

Status Enquiry			75

Facility Acknowledge		6A

Facility Reject			72

Register 			64

Elements:

Bearer Capability		04

Cause 				08

Connected Number		0C

Call Identity			10

Call State			14

Change Status			01

Channel ID			18

Progress Indicator		1E

Network Specific		20

Signal				34

Calling Party			6C

Calling Party Subaddress	6D

Called Party			70

Called Party Subaddress	71

Redirecting Number		74

Transit Network		78

Restart Indicator		79

Low Layer Capability		7C

High Layer Capability		7D

User-User			7E