Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

3.2.2.2 	ShoreTel TCP and UDP Port Usage

Table 3-2 and Table 3-3 provide information about the ports ShoreTel  devices and applications use to communicate with other ShoreTel  devices and applications.

Table 3-2 	Port Usage Part 1

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Switch | IP Phone | Communicator | Softphone |
| Switch | Call Control | Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 | MGCP UDP 2427 |  | MGCP UDP 2427 |
|  | Media Stream | RTP UDP 5004 (dynamic: 1024- 65535) | RTP UDP 5004 (dynamic: 1024-65535) |  |  |

42

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-2 	Port Usage Part 1 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Switch | IP Phone | Communicator | Softphone |
| Service Appliance | Call Control | Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 |  |  |  |
|  |  | HTTP Web share TCP 80 HTTPS Web share TCP 443 |  | Instant Messaging XMPP TCP5222 Presenter TCP 443 TCP 80 |  |
|  | Conference Bridge Integration |  |  | HTTP- Configuration Control TCP 80 |  |
|  | Media Stream | RTP UDP 5004 Configurable dynamic default: 10000- 10550) | RTP UDP 5004 Configurable dynamic default: 10000- 10550) |  | RTP for SIP UDP 1024- 65535 |
| Service Appliance | Media Stream V Switch only |  |  |  | NTP UDP 123 |
| IP Phone | Call Control | MGCP UDP 2727 |  |  |  |
|  | Media Stream | RTP UDP 5004 if dynamic 10000-10550 | RTP UDP 5004 [if dynamic 10000-10550 BB to Phone UDP 5554 |  | UDP 5004 RTP if dynamic: 10000- 10550 |

ShoreTel 13.3 	43

Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

Table 3-2 	Port Usage Part 1 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Switch | IP Phone | Communicator | Softphone |
| Communicator | Video |  |  | CMCP server TCP 5464 CMCP client TCP 5465 |  |
|  | Media Stream | Dynamic configurable default 1000- 10550 |  |  |  |
| SoftPhone | Call Control | MGCP UDP 2727 |  |  |  |
|  | Media Stream | RTP UDP 5004 if dynamic 1000- 10550 | RTP UDP 5004 if dynamic 10000-10550 |  | RTP UDP 5004 if dynamic 10000- 10550 |
| DVM Server | Call Control | RPC - NCC TCP 1024-65535 MGCP - Media proxy UDP 2427 Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 |  | Remote TAPI TCP 1024-65535 |  |
| DVM Server | Media Stream | RTP UDP 5004 [if dynamic 1024- 65535] | RTP UDP 5004 [if dynamic 1024-65535] |  |  |
|  | RPC Connection Negotiation | Port Mapper RPC TCP 111 UDP 111 |  |  | RTP UDP 5004 [if dynamic 1024-65535] |
|  | Configura- tion Control | Firmware download TCP 1024-65535 |  |  |  |

44

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-2 	Port Usage Part 1 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Switch | IP Phone | Communicator | Softphone |
| HQ/Director Server | Call Control | RPC - NCC TCP 1024-65535 MGCP - Media proxy UDP 2427 Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 |  | TCP 1024-65535 Remote TAPI |  |
|  | Media Stream | RTP UDP 5004 [if dynamic 1024-65535] | RTP UDP 5004 [if dynamic 1024-65535] |  |  |
|  | RPC Connection Negotiation | Port Mapper RPC TCP 111 UDP 111 |  |  |  |
|  | Configura- tion Control | Firmware download TCP 1024-65535 |  |  | RTP UDP 5004 [if dynamic 1024-65535] |
| HQ/Director Server | Maintenan ce | Diagnostic ipbxctl TCP 5555 | TCP 5555 Diagnostic phonectl |  |  |

ShoreTel 13.3 	45

Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

Table 3-2 	Port Usage Part 1 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Switch | IP Phone | Communicator | Softphone |
| Other | Call Control | SIP UDP 5060 |  |  |  |
|  | Media Stream | RTP - for SIP UDP 1024-65535 |  |  | RTP UDP 5004 [if dynamic 1024-65535] |
|  | Configura- tion Control | DHCP Client UDP 68 | DHCP Client UDP 68 |  |  |
|  | Mainten- ance | Telnet TCP 23 SNMP UDP 161 | Telnet TCP 23 |  |  |

46

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-3 	Port Usage Part 2

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| Switch | Call Control | Location Service UDP 5440 Call Control UDP 5441 DRS UDP 5442 Bandwidth Manager UDP 5443 Admission Control UDP 5445 | RPC - NCC TCP 1024-65535 MGCP - Media proxy UDP 2727 | RPC - NCC TCP 1024-65535 MGCP - Media proxy UDP 2727 | SIP UDP 5060 |
|  | Call Control: Softswitch |  | Location Service UDP 5440 | Location Service UDP 5440 |  |
| Switch | Call Control: Softswitch |  | Call Control UDP 5441 DRS UDP 5442 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS keepalive UDP 5446 | Call Control UDP 5441 DRS UDP 5442 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS keepalive UDP 5446 |  |
|  | Call Control: TMS RPC |  | NCC Event port TCP 5457 SUNRPC Broadcast UDP 5458 MSRPC TCP/UDP 5500- 5600 | NCC Event port TCP 5457 SUNRPC Broadcast UDP 5458 MSRPC TCP/UDP 5500- 5600 |  |

ShoreTel 13.3 	47

Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| Switch | Media Stream | RTP UDP 5004 Configurable dynamic default: 10000- 10550) | RTP Static UDP 5004 RTP Configurable dynamic default: 10000- 10550) | RTP Static UDP 5004 RTP Configurable dynamic default: 10000- 10550) | RTP - for SIP UDP 1024- 65535 |
|  | RPC Connection Negotiation |  | Port Mapper RPC TCP 111 UDP 111 | Port Mapper RPC TCP 111 UDP 111 |  |
|  | Configuratio n Control |  | FTP CTL - Boot files TCP 21 FTP DATA - Boot files TCP 20 | FTP CTL - Switch Boot TCP 21 FTP DATA - Switch Boot TCP 20 | DHCP Server UDP 67 |
|  | Database V Switch only |  | MYSQL TCP 4306 MYSQL Config TCP 4308 | MYSQL TCP 4306 MYSQL Config TCP 4308 |  |
|  | Transport V Switch only |  | CDS TCP 5432 | CDS TCP 5432 |  |
|  | Maintenance |  |  |  | SNMP TRAP UDP 162 |

48

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| Service Appliance | Call Control | Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 | Softswitch Location Service UDP 5440 Call Control UDP 5441 DRS UDP 5442 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS Keepalive UDP 5446 | Softswitch Location Service UDP 5440 Call Control UDP 5441 DRS UDP 5442 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS Keepalive UDP 5446 |  |
|  | Call Control: TMS RPC |  | NCC Event port TCP 5457 SUNRPC Broadcast UDP 5458 MSRPC TCP/UDP 5500- 5600 | NCC Event port TCP 5457 SUNRPC Broadcast UDP 5458 MSRPC TCP/UDP 5500- 5600 |  |
|  | Configura- tion Control |  | FTP CTL - Boot files TCP 21 FTP DATA -Boot files TCP 20 | FTP CTL - Boot files TCP 21 FTP DATA -Boot files TCP 20 |  |
|  | Database |  | MYSQL TCP 4306 MYSQL Config TCP 4308 | MYSQL TCP 4306 MYSQL Config TCP 4308 MYSQL CDR TCP 4309 |  |
|  | Transport |  | CDS TCP 5432 | CDS TCP 5432 |  |

ShoreTel 13.3 	49

Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| Service Appliance | Media Stream | RTP UDP 5004 Configurable dynamic default: 10000- 10550) | RTP Static UDP 5004 RTP Configurable dynamic default: 10000- 10550) | RTP Static UDP 5004 RTP Configurable dynamic default: 10000- 10550) | RTP for SIP Endpoints UDP 1024- 65535 |
|  | Collabora- tion Management (CMCA) | (Web Share) Ping Sync UDP/TCP 5450 |  |  | Ping Sync/ UDP 5450 (CMCA) to other Service Appliance TCP 5450 |
|  | Instant Messaging | UDP 5451 UDP 5463 TCP/UDP 5466 | Any HTTP TCP 80 Web-proxy, session manager TCP 5449, 5469 | Any HTTP TCP 80 Web-proxy, session manager TCP 5449, 5469 |  |
|  | IM XMPP |  |  |  | TCP/UDP 5466 message forwarding |
|  | STTS -NCC STTS - Media |  |  | TCP 5467 TCP 5468 |  |
|  |  |  |  |  | NTP UDP 123 DHCP Server UDP 67 SMTP Trap UDP162 SMTP TCP 125 DNS TCP/UDP 53 |

50

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| IP Phone | Call Control |  |  |  |  |
|  | Media Stream |  | RTP UDP 5004 if dynamic 10000-10550 FTP TCP 21 | RTP UDP 5004 if dynamic 10000-10550 FTP TCP 21 |  |
|  | Configura- tion Control |  | FTP CTL - Config TCP 21 FTP DATA - Config TCP 20 ICMP PING | FTP CTL - Config TCP 21 FTP DATA - Config TCP 20 ICMP - PING UDP 5004 RTP [if dynamic 1024-65535] | DHCP Server UDP 67 SNTP UDP 123 |
| Communicato r | Call Control |  | MS RPC - Remote TAPI TCP/UDP 5500- 5600 Remote TAPI Event TCP 1024-65535 | MS RPC - Remote TAPI TCP/UDP 5500- 5600 Remote TAPI Event TCP 1024-65535 |  |
|  | Configura- tion Control |  | CAS TCP 5440 CAS TCP 5447 Web-proxy, Mgmt API TCP 5449, 5469 | Web client, Online help TCP 80 HTTP CAS TCP 5440 CAS TCP 5447 Web-proxy, Mgmt API TCP 5449, 5469 |  |
|  | RPC Connection Negotiation |  | Port Mapper RPC TCP 135 MS | Port Mapper RPC TCP 135 MS |  |

ShoreTel 13.3 	51

Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| Communicato r | Web |  | Web-proxy, WCM options page/Mgmt API TCP 5449, 5469 Session Mgmt AD/non-AD login auth & Web content TCP 80 CAS TCP5447 | Web-proxy, WCM options page/Mgmt API TCP 5449, 5469 Session Mgmt AD/non-AD login auth & Web content TCP 80 CAS TCP5447 |  |
|  | Mobile |  | MCM Server TCP 80 | MCM Server TCP 80 |  |
|  | Instant Messaging | XMPP TCP 5222 |  |  |  |
|  | Conference Bridge Integration | HTTP Configuratio n Control TCP 80 Presenter TCP 443 TCP 80 |  |  |  |
| SoftPhone | Media Stream | RTP UDP 5004 if dynamic 10000-10550 | RTP UDP 5004 if dynamic 10000-10550 | RTP UDP 5004 if dynamic 10000-10550 |  |
|  | Configura- tion Control |  |  | HTTP TCP 80 |  |

52

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| DVM Server | Call Control |  | MS RPC - DTAS/ TMS TCP 1024-65535 Call data TCP 5441 Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS keepalive UDP 5446 | MS RPC - DTAS/ TMS TCP 1024-65535 MS RPC - DB access TCP 1024-65535 Call data TCP 5441 Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS keepalive UDP 5446 |  |
|  | Media Stream |  |  |  |  |
|  | RPC Connection Negotiation |  | Port Mapper MS RPC TCP 135 | Port Mapper RPC TCP 111 UDP 111 MS RPC TCP 135 |  |
|  | Maintenance |  |  |  | RPC - Quicklook TCP 1024- 65535 |
|  | Distributed Voice Mail |  | SMTP - Voice Mail transport TCP 25 | SMTP - Voice Mail transport TCP 25 |  |
|  | Voice Mail Notification |  |  |  | SMTP TCP 25 |

ShoreTel 13.3 	53

Planning and Installation Guide 	Chapter 3: Network Requirements and Preparation

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| HQ/Director Server | Call Control |  | MS RPC - DTAS/ TMS TCP 1024-65535 MS RPC - DB Notify TCP 1024-65535 Call data TCP 5441 Location Service UDP 5440 Call Control UDP 5441 Bandwidth Manager UDP 5443 Admission Control UDP 5445 DRS keepalive UDP 5446 |  |  |
|  | Media Stream |  |  |  |  |
|  | RPC Connection Negotiation |  | Port Mapper RPC TCP 111 UDP 111 MS RPC TCP 135 |  |  |
|  | Maintenance |  | RPC - Quicklook TCP 1024-65535 |  |  |
|  | Distributed Voice Mail |  | SMTP - Voice Mail transport TCP 25 |  |  |
|  | Voice Mail Notification |  |  |  | SMTP TCP 25 |

54

Chapter 3: Network Requirements and Preparation 	Planning and Installation Guide

Table 3-3 	Port Usage Part 2 (Continued)

| Originating Device | Traffic Type | Destination Device |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Service Appliance | DVM Server | HQ/Director Server | Other |
| HQ/Director Server | CDR |  |  |  | CDR archive on remote server TCP 3306 - |
| Other | Configura- tion Control |  |  | HTTP - Director TCP 80 |  |
|  | Maintenance |  | HTTP - Quicklook TCP 80 |  |  |

3.2.2.3	Bandwidth in the LAN

For LAN calls using the voice switches, 10 msecs of voice samples are encapsulated in a Real Time Protocol (RTP) packet before  being transmitted onto the LAN. For IP phones and SoftPhones, 20 msecs of voice samples are encapsulated in an RTP packet before being transmitted onto the network.

The protocol overhead consists of 12 bytes for the RTP header, 8 bytes for the UDP header, 20 bytes for the IP header,  and 26 bytes for the Ethernet framing. When ADPCM voice encoding is used, an additional 4 bytes are added to the voice data for decoding purposes. This yields an effective LAN bandwidth as shown in Table 3-4.

Table 3-4 	LAN Bandwidth—Bytes

|  | Linear Broadba nd | Linear | G.711 | ADPCM (G.722) | G.729a | G.729a |
| --- | --- | --- | --- | --- | --- | --- |
| Voice data (10 msec) | 320 | 160 | 80 | 40+4a | 20 (20 msec)b | 30 (30 msec) |
| RTP header | 12 | 12 | 12 | 12 | 12 | 12 |
| UDP header | 8 | 8 | 8 | 8 | 8 | 8 |
| IP header | 20 | 20 | 20 | 20 | 20 | 20 |
| Ethernet header and framingc | 26 | 26 | 26 | 26 | 26 | 26 |

ShoreTel 13.3 	55