*=. (as in 192.168.)

# = Toggle to next screen and accept

Speaker= backspace but not back to previous setup screen

Plug in Phone

Enter password (1234)

Clear all values enter #

DHCP select # for OK, or * to toggle NO, then pound

FTP = your ShoreTel server address (ie 192.168.8.5)

MGC select # for not set, let the server give it the switch information

SNTP is your time server, if you do not have one use 192.5.41.41

Then hit # key about 8 more times to save values and let phone reboot.

If the phone has never been booted it will take about 4.5 minutes the first time.