Ipbxctl –telneton 172.16.3.TELNET CAN ONLY BE DONE FROM THE HQ SERVER

cd\

For 32bit

cd pro*\sh*\*er

For 64Bit

cd *)\sh*\*er

ipbxctl -telneton ipaddress

password: ShoreTel (for older switches skip this step)

telnet ipaddress

log: anonymous

pass:ShoreTel

command example:

lsp_ping "ip address"

bootChange  (this will let you edit the ip info)

FOR PHONES

phonectl -pw 1234 -telneton 192.168.1.5

you need to authenticate with a password. default is 1234

printSysInfo   (this will display all info inside the phone)

BurnFlash 1U

CD C:\prog*\Shore*\*ver

burnflash –s 192.168.1.xx

This will re-flash the switch with new firmware

Rom Update

- Enter a telnet session and fully login

- Type gotoshell

- Enter the command “uboot_update” the switch will pause and update the rom

- type exit

- done