The audio levels for the handset, headset and speaker are set to default values when the phone boots.  These values can be overridden by configuration file settings.  The phone supports multiple configuration files, and the last setting encountered in a configuration file wins.  The base configuration file for each phone type is created by the installer, so although new values may be set in these files, they will be lost when the system is upgraded.  Customized audio levels should be set in customized configuration files rather than the base configuration files.    Below is a table showing phone name, model and file names:

----------  --------  -------------  -------------  -------------------

ShorePhone  Name on   Model Name on  Base           Custom

Name        Top of    Barcode Label  Configuration  File Name

Phone     Underside of   File Name      c:\Inetpub\ftproot\

Phone

----------  --------  -------------  -------------  -------------------

210        210         S1         shore_s1.txt      s1custom.txt

210A       210         S1A        shore_s1a.txt     s1custom.txt

530        530         S2         shore_s2.txt      s2custom.txt

560        560         S6         shore_s6.txt      s6custom.txt

There are four sets of audio levels that can be custom configured:  headset, handset, speaker and ringer.  Below are the default values for each type:

HandsetLevels    5157,183,258,365,516,728,1029,1631,2303,3254,4596,917,13

HeadsetLevels    6144,183,258,365,516,728,1029,1631,2303,3254,4596,1375,13

RingerLevels     130,258,410,649,1029,1631,2584,4096,6492,10284

SpeakerLevels    4096,258,410,649,1029,1631,2584,4096,6492,10289,16306,13

Here are the parameter definitions:

HandsetLevels TxGain,RxGain1,RxGain2,RxGain3,RxGain4,RxGain5,RxGain6,RxGain7

RxGain8,RxGain9,RxGain10,SideTone,Handset DTMF Attenuation

HeadsetLevels TxGain,RxGain1,RxGain2,RxGain3,RxGain4,RxGain5,RxGain6,RxGain7

RxGain8,RxGain9,RxGain10,SideTone,Headset DTMF Attenuation

RingerLevels  RxGain1,RxGain2,RxGain3,RxGain4,RxGain5,RxGain6,RxGain7

RxGain8,RxGain9,RxGain10

SpeakerLevels TxGain,RxGain1,RxGain2,RxGain3,RxGain4,RxGain5,RxGain6,RxGain7

RxGain8,RxGain9,RxGain10,SideTone,Speaker DTMF Attenuation

The "TxGain" parameter sets the level of the audio transmitted from the phone onto the network.  The "RxGain" values correspond to each of the 10 volume setting levels shown when one adjusts the volume on the phone.  "SideTone" is the audio picked up from the microphone and transmitted to the speaker (speakerphone, handset or headset speaker) that provides feedback to the user that the phone is working speaking.  Customer experience has shown that sometimes the RxGains are not high enough and need to be customized for an individual system.  SideTone gain is very subjective, and is sometimes lowered and sometimes raised.  DTMF Attenuation sets the receive DTMF level that the phone user will hear.

The Plantronics CS50 wireless headset provides its own side tone, plus inserts 20 ms delay between the headset and the phone, which causes some people to say they hear echo when using the Plantronics headset, and in this case headset side tone may need to be reduced.   When you speak, Plantronics does reduce the gain on the speaker by -24dB, but when you stop speaking, the gain is turned up, and you can hear the last little bit echoed in your ear.

For example, inserting the following line in "s2custom.txt" or "s6custom.txt" and rebooting the phone increases the headset volume settings 11 dB at the highest setting, which may be required in noisy environments:

HeadsetLevels 6144,183,258,409,649,1028,1630,2899,6491,10288,14333,1375,13

The same levels could be used for "HandsetLevels", and also for the 210 phone.  Inserting the following line reduces the headset side tone by 9.5 dB, which some CS50 users prefer:

HeadsetLevels    6144,183,258,365,516,728,1029,1631,2303,3254,4596,460,13

Minimum gain values are 0 and maximum are 32536.  Setting values very high may saturate the speaker and create poor sound quality.

N.B.:  You can change gains on an individual phone by changing the configuration files and then rebooting that phone.  But eventually all phones on the system will obtain these values when they eventually reboot.  We do not currently have the ability to individually configure ShoreTel IP phones.